# Mine Signs ID Label Fitment Guide

## Items Required –

- Quick drying evaporative cleaning product such as Prepsol.

- Clean rags to clean surface

- Masking tape (50mm)

- Scissors

- Squeegee

- Tape Measure

A clean surface is key to full adhesion for long service life of the ID Label

## Application Steps –

| Step 1 | Use Prepsol and rags to thoroughly clean surface. |  |
| --- | --- | --- |
| Step 2 | Use measuring tape to mark label position. |  |
| Step 3 | Position sign without removing the backing paper and use a tab of masking tape at each side on the vertical edge to hold the ID Label in position. |  |
| Step 4 | When you are happy with the position of the ID label, place a strip of masking tape vertically across the full length of the ID Label just past centre to ensure the label doesn’t move. |  |
| Step 5 | Cut the masking tape tab on the right side edge of the sticker so you can fold the sticker back toward you half way. |  |
| Step 6 | Peel the backing paper off the label, cutting the excess paper off with your scissors. |  |
| Step 7 | Lay the label back on to the surface carefully realigning your masking tape to ensure your label is straight. |  |
| Step 8 | Holding the right hand edge of the label off the surface, use your squeegee and start rubbing the surface using a centre out motion toward the right hand edge. |  |
| **Holding the right hand edge of the label off the surface, this ensures that as you squeegee out, it releases any air bubbles from getting trapped** |  |  |
| Step 9 | Remove the centre strip of masking tape and repeat steps 3 through 7 only this time on the left hand side of the ID Label. |  |
| Step 10 | Remove all excess masking tape and squeegee the outside edges ensuring full adhesion contact is made. |  |